Industrial Pharmacy

Production

Small Scale		                   	Large Scale

Private (Special)           Hospitals

Definition of industrial pharmacy :- It is a branch of pharmacy which deals with process of production of pharmaceutical preparations in the way that the end product should have the following properties

- Effective with the least side effect .

- Have good taste , odour , flavour and shape .

- Stable for the specified life .

- Non – toxic .

Or may be defined as – that science and technology of producing pharmaceuticals (medicinal sub–es and their dosage)  efficiently and economically on the commercial scale .

Reasones for large scale production :-

- Economy .

- Accuracy .

- Greater scope .

Industrial pharmacy may be classified into the following areas .

- Pharmaceutical engineering including :-

- Unit operations .

- Unit processes .

- Pharmaceutical packaging .processes developments .

- Pharmaceutical packaging .

- Pharmaceutical administration .

Unit operation Basics :-

- Fluid flow .

- Heat transfer .

- Mass transfer .

  - The flow of fluid in pipette is not at the same velocity (not constant) and is effected by the diameter of pipette , velocity of the fluid , density of fluid , the fluid viscosity and the pressure applied .

Reynold found that fluid flow condition , where effect by :

- Pipe diameter D .

- Fluid velocity V .

- Fluid density   P .

- Fluid viscosity M .

- Reynold Number  Re  .

Reynold Law     Re = PVD/M

In the pipe the flow of the fluid is :

- Turbulent at the center .

- Decreasing as it becomes far from the center .

- Until it becomes finally a stationary full on the wall .

2- Heat transfer:-Process by which energy in the form of heat is exchanged between bodies or parts of the same body at different temperatures heat can be transferred by three processes conduction , convection and radiation .

Conduction:- is the transfer of heat along a solid object , the process that makes the handle of a poker hot , even if only the tip is in the fireplace

Convection:-Transfer of heat through exchange of hot and cold molecules , this is the process through which water in a kettle becomes uniformly hot even though only the bottom of the kettle contacts the flame .

Radiation:-  Radiation is the transfer of heat from hot body via electromagnetic (usually in frared) variation to a receiving body which absorbed the heat ; This is the principal mechanism through which a fireplace warms a room .

Mass transfer:- Is the transition of material from one phase to another .

Ex: In distillation the liquid converted to vapor  then to liquid purification .

Extraction of solid material then converted it to solution then to solid for purification .

Partical  Size Processing:-

The partical of any solid is not of equal size and shape ; it is important to describe the particle size  definition : the size of each individual particale of a solid , this can be either the diameter of the circular partical or the mean of the three dimensions  .

The properties of  powders may be influenced by :-

- The particle size .

- The surface area , which in turn depends on particle size .

The factors by which the particle size may influenced :-

  - Suspension – In very small particles a " clay like " sediment is difficult to re-disperse , while in large particles the rapid sedimentation will make the suspension difficult to use and to measure the accurate dose .

  - Mixture of solid : In a mixture of powders of different particle size aggregation may resulting in separation of large particles .

  - Solid / Fluid preparation : (Like filtration and sedimentation)

  - Granule size : To manufacture tablets in high quality a car ful control of granule size is required .

  - Process factors : The flow character of a powder is affected by the partical size , loss of fines as dust , risk of explosion e.g KCLO3 .

The surface area :-

The " outer " surface area of any two dimensional particles , or the " total " surface area including cracks , fissures and pores in particles . The surface is cudweed by adding  together the areas of all the faces of the object .

Decreasing the particle size of a powder  will be greatly in creased in its surface area .

The surface area may influenced by :

    - Mass transfer processes – Operations (such as solution extraction or drying) are influenced directly on surface area and indirectly by particle size .

    - Sorption : Both absorption and adsorption are dependent on the particles surface area .

Partical size analysis :-

A classification of particles according to their size is needed because not all of the particles in a compound are the same size .

There are numerous methods for particle size analysis the methods discussed below may be used for powders with a particle size less than 1000 .

It is important to choose a representative sample to get the correct result . This requires to notice that .

  - a sample from the " top " of a container is not sufficient because aggregation may have occurred during transport .

  - A- batch dilled in many containers may show that one container may not be typical to the entire batch , since aggregation may take place during .

Methods for particle size analysis:-

- Non fractionation methods that evaluate an entire population of particles

- Microscopy (optical , electron)

- Light scattering (dynamic , photon correlation laser diffraction)

- Electro zone  sensing .

- Photo zone sensing .

- Fractionation methods that use physical techniques to separate particles on the

- Basis of size .

- Sieving .

- Chromatography .

- Sedimentation .

- Gravimetric Methods .

- Sieving .

- Fluid classification .

- Water classification method .

Sieving:- a sieve separates wanted / desired  particles from unwanted using a tool such as a net . A strainer – is a type of sieve typically associated with separating liquids from solid the size of sieves can be defined by one of 2 ways mesh number : The number of openings per linear inch of mesh (2.54cm) . For instance a mesh number 4 mesh will have 4 openings per linear inch .

Aperture size : Which is the nominal size in mm. to exceeding 1m.m and in microns for smaller apertures .

| Mesh no. | Aperture size | Meshes كلما زاد عدد الفتحات كلما صغرت الفتحات (Aperture) |
| --- | --- | --- |
| 4 | 4 mm |  |
| 6 | 2.8 mm |  |
| 10 | 1.7 mm |  |
| 22 | 710 µ |  |
| 44 | 355 µ |  |
| 300 | 513 µ |  |

Factors affecting sieving process :-

  - Electrostatic charges : Some particles have different charges arise during attraction between them leading to becomes static's or give  vise to segregation .

  - Humidity : Hydroscopic materials may aggregate in high humidity conditions .

  - Particle shape : Only spherical particles pass through the mesh easily and as particles departs from the spherical shape difficulties arises as with fibres or plate like particles , which may tend to block the aperture .

  - The shape of the aperture circular and square apertures are the best , rectangular shape may affect the result of sieving .

  - Disturbance of the bed : Sieving requires some disturbance of the bed of particles e.g. vibration or shaking . This may be done manually and sieves may be supplied with brush or other means of disturbing the powder during sieving .

  - Time : Sieving is impossible to be continued to completion especially with regard to the end point thus for particle size analysis sieving may be done to a standard time or until the weight of powder passing through the sieve in unit time is less than a certain percentage of the original sample . Prolonged sieving  may lead to the formation of more fine particles by attrition of particles between each other or against the sieve classification of powders in the sieve size :

- Coarse  powder 10/44 – All the powder passes through number 10 mesh and not more than 40% passes the number or through number 44 mesh .

- Moderatly coarse powder : 22/60 All the powder passes through number 22 mesh and not more than 40% passes through number 60 mesh .

- Moderatly fine powder 44/85 .

- Fine powder 85

- Very fine powder 120 .

- Ultra fine powder : 90% of the powder having particle size less than 5/11 and no partical  .

SIEVING :-

In the sieving process powder can be divided into fractions by passing through series of sieves of diff. siles . The coarsed are placed on the top of sieve of the final sieves by this method the powder is passed successfully over the final sieves and a series of fraction obtained .

No force should be used in the process except on the granules to accelerate the sieving process . certain methods can be used .

1- Agitation : In agitation the set of sieves may be agitated by one of the following methods .

    - Oscillation – Where the set of sieves ore mounted to on oscillatory fram which moves back and forth .

    - Vibration .

2- Gyration – It is a rounded movement .

By sieving the powder may be classified to :-

| Powder | Name | Ail passes through mesh NO. | Not more than 40% passes through mesh NO. |
| --- | --- | --- | --- |
| Coarse | 10/44 | 10 | 44 |
| Moderately coarse | 22/60 | 22 | 60 |
| Moderately fine | 44/85 | 44 | 85 |
| Fine | 85 | 85 |  |
| Very fine | 120 | 120 |  |
| Ultra fine | 90% of the powder have Particle size Less than 5/4 |  |  |

The result of sieving can be either tabulated or presented graphically .

| Particle size | % |  |  |
| --- | --- | --- | --- |
| 0.2 or Less | 12 | 0.8+1 | 2 |
| 0.2 – 0.4 | 48 | 0.6 – 0.8 | 11 |
| 0.4 – 0.6 | 27 | 0.2 – 0.4 | 48 |
| 0.6 – 0.8 | 11 | 0.2 or less | 48 |
| 0.8 – 1 | 2 | 0.2 pr less | 12 |

Fluid classification :-

  - water sedimentation .

  - Air sedimentation .

Definition – The movement of particles in the sub – sieves in a fluid is the bases of particles size analysis by fluid classification method .

The movement of a spherical particles in a fluid can be expressed by stock's Law .

T=29×p2dp−dL.kV

Where -

T- is the time for the particle move a stated distance .

p- particle size .

p- particle size .

dp- density of powder .

dl- density of fluid .

v- viscosity of the liquid .

k- constant .

Liquid sedimentation:-

A 2-5% suspension of the insoluble powder in a suitable liquid is prepared and filled in a long measuring cylinder placed on table to prevent any movement . sample are with – drawn from a constant Level (for ex . 10 cm.) from the top of the Liquid at content intervals and analyzed  no air bubbles should be present in the suspension .

Air separation:-

Ex- cyclo separation , this is a cylindrical vessel with a conical base an inlet on one side and a central fluid outlet at the top the suspension in a suspension in a fluid is introduced at high speed where it moves in rotary movement within the vessel this causes the particles to settle down by the action of centrifugal force and collected from the lower discharge (outlet) while the fluid is removed from the upper fluid out let .

Elutriation :- practically in elutriation can be practiced by using a vertical column supplied with inlet in the bottom and a solid outlet at the base and fluid outlet near the top . The suspension is introduced through the inlet at high speed , coarse particles will settle down and colleted at the base while the fluid and fine powder or particles move upward and may be colleted from the upper outlet by using series of columns connected in such a way that the fluid Leaving the first column used as a feed for the second column by this method the solid can be damaged into fractions of decreasing particles size in interior rate they travels upwards , while the particles settle down depending on it's size .

Microscopic Method :- This is a very effective procedure , there many factors effecting the results , including particle shape , position , aereggation , sampling method and speed of examination .

Mean particle size measurement : It is the sum of particles sizes divided by its number .

In the previous methods either the individual particle size (as in the microscope method) or series of groups of particles size are obtained . In the following method a single method is obtained giving a mean value for the particle size of the sample .

A- Adsorption or surface method .

Powder under certain circumstances will adsorpe mono– molecular layer of a fluid (Liquid or gas) , the measurement of the fluid adsorped will give a value for the surface area of the powder which is intern a mean of it's particle size .

Fluid I/P                                                           O/P

B- permeability method :-

When a fluid is passed through a packed bed of a solid , the rate of flow may be expressed by Darey's Law .

dvdt=KAΔPμL

Where v- Is the volume of fluid passing .

t – Time .

A – area .

∆P – pressure drop .

µ – viscosity .

L – thickness of bed .

K – permeability constant of the bed .

The constant which is the permeability of the bed is related to the mean particle size .

Conductivity Method :-

One of the most recent particle size analysis proc – endures to be developed is the coulter counter in which the particles are suspended in an electrically conductive fluid . The suspended fluid is passes through a suitable a suitable aperture with an immersed electrode on either side and the particles concentration is arranged so that only the particles travel through the aperture at a time , It is used for inject able solution , the instrument known as coulter counter .

SiZE REDVCTION :-

Definition – Is the operation used to reduce a material to smaller pieces , to coarse particles or to powder , the reasone for including in the definition three terms suggesting different degree of size reduction will be apprent when the various methods of attaining these objects are considered . The term "size reduction" used to explain a mechanical process used to reduce the size of a solid material , whether natural or industrial , whether crystalline , fibrous of granules .

Objectives :-

- To produce small pieces , thus to facilitate powder mixing , stability of suspension and increase the rate of drying .

- To increase the surface area , thus increase the rate of adsorptive properties and effect the rate of absorption .

- To crush drugs and expose cells before extraction .

- To reduce the bulk or the volume of drugs .

Factors affecting size reduction :-

In single product project , the particle size may be easily standardized as in Flour or ciment industries while in pharmaceutical      industry , where a Large verities of materials are used . for ex . chemical and drugs which may hard , fibrous or spongy . The methods of size reduction are numerous and may be influenced by :

- Hardness : It is a surface property and differs from strength since a material like chocke may be hard but brittle , thus it can be easily milled , a special hardness scale has been designed known as "Moh's scale" which gives material no . from 1-10 . materials from no . 1-3 are soft , Materials which can be marked with finger nails , from 3-7 are intermediate and above 7 hard , materials which can not be marked with a knife blade .

- Toughness : This is a more important property than hardness , thus a soft and Tough material Like rubber is more difficult to break than a hard and brittle material Like chocke , the toughness is affected by .

  - Moisture content – Especially in fibrous grain a green twig is tough while the dry gone is brittle .

  - Cooling – may allow tough materials like rabber to break easily .

  - Abrasiveness – hard materials which are abrasive may lead to contamination during grinding .

  - Stickness – some materials tend to adhere to the surface of the grinding machine or to sieve .

[certain factors may assist in reducing stickness] :-

- Gumming and resinous materials , some materials better grinded in cooled mills .

- Complete dryness .

- Adding inert substances : ex . the addition of small amounts of kaolin may decrease the stickness property of sulfur .

The energy required in size reduction :-

The mechanism of size reduction is very complicated and only a very small amount 2-5% of the energy put into machine affect the size reduction , the remainder being lost in many ways like deformation of particles , friction between particles , friction in the machine , heat generation and noise .

Mechanism of size reduction :-

- cutting …… cutter mill .

- compression ….. Roller mill .

- Impact …… Hammer mill .

- Attrition …… File .

- Impact and attrition …. Ball mill , fluid energy mill .

- cutting – By cutting the material is cut by means of sharp blades (plates) knives , or scissors .

- compression – By compression the material is crushed by applying pressure like a motar and pestle or crusher .

- Impact – By this method either the material remains stationary and is hit by a moving object at a high speed or moving particle strike a stationary surface ex. Hammer .

- Attrition – By attrition the material is subjected to pressure but the surfaces are moving relative to another e. g. file .

Cutting mill and cutting crusher :-

It is a coarse crusher used to obtain a coarse material , this is used for hard and fibrous drugs , such as roots , woods and peals , before extraction the mill consist of .

- casing with a stationary knives , an upper hopper to feed the drug and a Lower screen to receive the finished product .

- a central rottor on which an o. Of knives are attached as the rottor rotates the drug crushed between the knives .

Roller mill :- one of the fine crusher used to crush seeds before extraction , the mill consist of two cylindrical rolls made of metal or stone , mounted horizontly and can be rotated on the longitudinal axis either .

- one of the roll is driven mechanically while the second runs free .

- both rolls are driven mechanically , the serene product gap between the rolls can be adjusted to control the size reduction required , the speed can be adjusted also .

Hummer mill :- An intermediate crusher which can roots and cry stalls , it consist of :-

- A stout metal casing , supplied with a screen al the bottom , to receive the finished product the inertial of the casing may be undulating to increase the efficiency of the process .

- A central shaft on which a No. of swinging hummers are attached , the hummers can swing out to a radial position or rotating the shaft , the hummers can be replaced by a rigid beater where the machine is named (Disintegrator) .

Ball mill :-

One of the fine crusher used widely in diff . industries as ciment and pharmaceutical plants , the mill consist of :

- A hollow cylinder of diff . sizes which rotate on its horizontal longitudinal axis , it can be made from metal , porceline or rubber .

- The cylinder contain a No of balls which fill 30-50% of the mill , the balls can be made from metal , porceline or quartz , and different sizes are available depending on the size of the cylinder .

If the cylinder is rotated at the correct speed . The size reduction is effected by the impact of particles between the balls and by. Attrition between the balls .

Factors affecting size reduction in ball mill The major one is the speed of rotation , At Low speed the balls gather and slide or roll over each other , a very negligible amount of size reduction occurs , At high speed the balls will be thrown out to the wall of the cylinder by centri- fugation and no size reduction occurs , At moderate speed (about 2/3 of the speed of centreifugation – correct speed) . the best size reduction occurs , since the balls are carried to the top of cylinder and then fall across the diameter of the mill m effecting size reduction .

| Advantages | Disadvantages |
| --- | --- |
| 1- It is useful for a wide verity of material both solid and wet grinding | 1- both cylinders and balls may worn away , contamination . |
| 2- It produces fine powder . | 2- when sticking mat. Are used the balls aggregate and caking may occur on the side of the mill . |
| 3- It is completely enclosed thus prevent . | 3- It is noisy . |

Fluid Energy Mill :- An ultra fine grinder , It is used where fine and potent powder are required Like anti- bodies , vitamins , sulphonamide , the mill consist of a Loop of pipe , the diameter of which depends on the hight of the Loop , at the bottom of the loop , a fluid Liquid (Gas , air or nitro gas) is injected at high pressure through nozelles giving rise to high velocity circulation in very turbulent condition , this allows the solid to be introduced into the screen from a hopper at the solid inlet , the main advantage of this type of mill are :-

  - The use of gas leads to cooling , thus it is useful for thermolabile materials .

  - For sensitive materials , inert gas may be used ,'

  - No abration or contamination occur .

Wet Grinding :-

Colloid mill (homogenizer) , It is an ultra fine mill and consist mainly of a rotar which rotates over stationary plate (stator) . both with conical working surfaces , resulting in size reduction of the product , which is feed from a hopper , at the of the machine , the mill may be water jugated to cool the product the rotor may work at different speed of the order of 3000 – 5000 r . p . m. and the clearance can be adjusted between 50 – 500 Micron .

-Mixing-

Mixing is used widly in nearly each operation or process and including a variety of materials whether Liquids , semi – solid or solid . mixing may be easy and simple when two misicible Liquids or a soluble solid and a solvent are used . But it becomes more difficult when in – soluble solids are suspended in a Liquid or two or more solids of diff . qualities and different particle sizes are mixed .

Definition : mixing is the operation in which two or more components are treated so that each particle lies as nearly possible in cont – act with a particle of each of the other ingredients , this theoretical deffinition may not practically and depend on the purpose of the product and the objective of mixing .

Objectives :-

  - simple physical mixture .

  - physical change .

  - Dispersions .

  - Promotion of reaction .

- simple physical mixture – This is a blend of two or more miscible Liquids or 2or more solids having uniform particles such mix . are of a high order of uniformity to ensure correct dosage .

- physical change – In this case a physical (not a chemical) change take place e. g. The solution of a soluble substances where mixing only accelerates a process which may .

- Dispersion – In dispersion two immiscible liquids emulsions or insoluble solid and a Liquid are mixed to form emulsions or suspension , In this case good mixing is required to reach homogeneity .

- Promotion of reaction – mixing encourage a chemical reaction to ensure a uniform product .

Types of mixtures :-

- positive mixture .

- Negative mixture .

- Neutral mixture .

Positive mixture :- These are irreversible . mixture where mixing takes place by diffusion and no work is required if time is unlimited no problems arrises in such mixture e. g. mixing of two miscible Liquids or gases .

Negative mixture :- These are mixtures of two immiscible Liquids or an insoluble solid and a Liquid (suspension) or (emulsion) : whose contagious mixing is required to reach homogeneity . Thus such mix .s are more difficult to form and require a higher degree of mixing .

Neutral mixture :- These are a static mixtures their components have no tendency to spontaneous mixing or segregation , neutral mix. Include diff. preparations Like paste , ointments and mixed powders . many factors effect the prep . of such mix. Including viscosity ., density , particle size , and vetting of solids in addition to other factors such as proportion of components and the order of mixing . mixing will be dealt with in three categories Liquids solids (powders) and semi- solids materials to be , dealt in pharmacy lying in the range from mobile Liquids to solids , wet and dry .

A – Liquid mixing :-

In general , the mixed condition not difficult to ach – ieve with two or more mobile Liquids or with mobile Liquids and solids although the Latter mixture is un – Likely to be permanent . mixing is usually performed with a mixing element e. g. a rotational device like an "impeller" which produces aproperate (good) flow of mixer .

The Flow pattern is influenced by :-

  - The form of the impeller paddle , anchor shaped , single or multiple propeller or Turbine .

  - The position of the impeller , wether high or low central or one side .

  - The shaft , wether vertical or inclined .

  - Shape of the container and the presence of baffles .

  - Liquid properties – The viscosity of the Liquid is very important and may influence the design and operation of the mixer .

e. g. A Liquid with low viscosity may be easy to mix with small impeller rotated at high velocity , while a high viscosity Liquid (paste) requires a Low speed rotation for an impeller with blades which scraps the sides of the vessels .

when 2 miscible liquids of different densities are to mixed , the mechanism of mixing can be represented by the following fig . in which we plot the time versus the degree of mixing .

Equipments :- A system for batch mixing commonly consist of 2 primary components .

- Tank or container .

- A mean of suppling energy to the system so as to bring about reasonably rapid mixing . power may be supplied to the fluid mass by means of impeller , air stream or Liquidver , Besides supplying power , these also serve to direct the flow material within the . vessel . Baffles , vanes or ducts are also used to direct the bulk movement of materials .

Impellers – can be distinguished on the basis of tune of flow pattern they produce or on the basis of the shape and pitch .

Propellers – characteristically produce flow paralel to their axes of rotation , where as turbines may produce either axial or tangential flow or a combination of these . propellers are most commonly and efficiently used at high speed in Liquids of relatively low viscosity .

Paddles also are employed as impellers and norm – ally operated at low speed 50 rom or less , their blades have a Large surface area in relation to the tank in which they are employed a feature that permits them to pass close to the tank walls and effectively mix viscous liquids or semi – solid .

Continuous mixing :-

It is used when very Large quantities of materials are to be mixed , this can be achieved by :-

In tube or pipe through which the materials flow and in which there is very little back flow or re cir collation occur .

In a chamber in which a considerable amount of materials are hold up and recirculation occur mainly used for heavy and large amount .

Solids Mixing :-

Since powders are generally neutral mixtures , thus powder mixing operation is different from that of Liquids , and the theory of solids mixing has not advanced mush beyond the most elementary of concepts and consequently is far benign that which has been developed for solids since mixed powders are generally sub divided to individual doses each should contain the correct proportions . For Q.C. sampling and sampling size becomes more important and the pharmacopaea gives a full details on sampling , on the other hand the particle size and particle size distribution are important since they largely determine the inagnitude of forces , gravitational and inertial , that can cause interparticulate movement relative to surface forces which resist such motion .

Particle density , elasticity , surface roughness , and shape also exert their influence on the bulk proper – ties of powders , of these , particle shape is perhaps the most difficult variable to describe .

Forces Acting in Multiparticulate solids systems Are essentially of 2 types :-

- those that tend to result in movement of two adjacent particles or groups of particles relative to each other .

- Those that tend to hold neighboring particles in a There are 3 principle mechanisms that are used in solid mixing which include .

- Diffusion – Redistribution of particles by random movement of particles relative to each other .

- Convection – movement of groups of adjacent particles from one place to another within the mixture .

- Shearing – change in the configuration of the ingredients through formation of slip planes in the mixture – Mechanism of powder mixing .

A static bed of particles under the influence of gravity will adopt a very close form known as Rham bohe deral packing in which the porosity is about 25% . Mixing demands movement of the particles , so that cubical packing is brought about , where the porosity is about 50% .

Mixing requires shear force which may brought about by :-

- Tilting the material beyond the angle of repose this is as the diffusion mixing . cohesive powders having high angle of about 50 "while easily" flowing powder having angle in the or der 25 .

- By using agitator arm or blast of air and this is known as shear mixing .

Factors Affecting The (Rate of Mixing) :-

- Material properties – This include the density , particle size , particle shape and particle attraction (the electrostatic charges) . when the densities of the materials are close to each other the pro cess of mixing and segregation is better .

- Proportion of materials – when the amount of mixture are identical the mixing operation is good (equal qualities are better) .

- Condition of mixing – This include mixer volume mixing mechanism , mixing time , handling the mixed product .

Mixing of Semi – solid :-

The mechanism of mixing semi – solid may show considerable variation depending on character of materials the Rheology may vary from Newtonion and non – Newtonion . on mixing an insoluble powder with Liquid , the following stages are observed :-

- Pellet and powder stage – In this stage some of the powder is converted to pellet embedded in a dry material .

- Pellet state – On adding more Liquid all the powder is converted to pellets .

- Plastic state – In this state the aggregate adheres . the mixture becomes homogenous and day Like .

- The sticky state – where the mixture become paste like with shiny surface .

- The Liquid state – On adding more Liquid the consistency decreases until a fluid stage is reached .

Note – care should be taken when soluble and insoluble powder are included .

Mixing Equipments

Liquid mixer            semi – solid mixer     solid mixer

(1) propeller mixer       1- Agitator mixer      1- Trough mixer

(2) Turbine mixer         or planatory              2- Tumbling

(3) paddle mixer            2- sigma mixer                  mixer

Propeller mixer :- it is the most widly used for mixing mobile Liquids , It consist of a cylinderical container supplied with a propeller similar to that used in marines . The mixing pattern is effected by the shape and size of the propeller , it's position in vessel and by the speed of operation , A propeller to container ration (1-20) operation at a high speed . about 8000 R.P.M is satisfactory for mobile Liquid – Aeration – air may ensure oxidiation this can be avoided by emersing the propeller deep in the liquid and by Asymmetry .

Asymmetry – This can be done be allowing the propeller shaft to be:-

  - off – set from the center .

  - mounted in an angle .

  - lnter the side of the vessel .

The shape of the vessel , may be other than cylinderical even this may give rise to dead spots in the corners .

The use of the two propeller on opposite pitch mount ed on the same shaft , and rotating at opposite direction known as push – pull may increase the efficiency of the mixer , The propeller mixer is not suitable for viscous liquid like Glycerin , caster oil and emulsions .

Turbine mixer :-In this mixer the propeller is re – placed by a circular disc impeller to which attached a No . of short straight or curved vertical blades the mixer is rotated at lower speed , the turbine mixer is useful for mobile Liquids , but can be dealt with more viscous Liquids , Like glucose or immiscible Liquids in the prep . of emulsion .

Paddle mixer :- In paddle mixer , the agitator used consist of flat bladdes or paddles attached to a vertical shaft which rotates at a low speed about 10 r. p. m. the bladdes are used for low viscosity Liquids while the paddles are used for viscous Liquids .

Size Reduction Equipments :-

Are used as parliamentary for small scale , but to obtain complete mixing of powder , the following mixer may be used .

1- Agitator mixer [Trough mixer] :-This consist of a trough supplied with an arm rotating to achieve mixing , The agitator is fitted with helical bladdes , this type of mixer is suitable for flowing materials with uniform partied size .

This type of mixer may be modified by :

  - using two bladdes in the form of the letter rotating at opposite direction , this is similar to the kneeder used in bakeries .

  - In stead of mechanical agitation an air stream may be used , the air flow should be intermittent and not continuous to prevent segregation .

2- Tumbling mixer :- This consist of a cylindrical vessel rotating on it's horizontal axis the vessel may be designed as the (V) or (Y) letter , Buffles may be added to facilitate mixing . Here the speed of rotation is very important , too low speed cause sliding only while too high speed will give rise to centrifuging . The method of charging the load may important and running the ingredients together into the mixer is the ideal method .

Semi – Solid Mixer :-

1- The agitator mixer :- used for Liquids and powders may be used for semi – solid but it is usually of a heavier constriction . It is supplied with plantory arm . may be used the following modification .

  - Agitator arm , may be designed to give a pulling or kneading.

  - The movement is arranged to clean the materials from the corners and sides of the vessel .

2- sigma arm mixer :-

It consist of a double trough shaped vessel supplied with blades having the shape of the Greek Letter sigma rotating towards each other , the two blades rotated at different velocities to facilitate mixing .

This type of mixer is suitable for heavy materials , such as the granules for tablets and ointments . The mixer can be enclosed and operated at reduced pressure to pre – vent air which may lead to oxidation. On the other hand the mixer may Jacketed for heating or cooling when necessary .

- Ultra sonix mixer:- The ultra sonic vibration are used now in certain types of mixers , specially for the prep . of emulsion , but such type are still used on small scale only .

- Magnetic stirrer :- On small scale the magnetic stirrer is used as a mean of mixing by transferring the electric power to magnetic field. The vessel is placed on a plate over area of magnetic field , followers are used in the vessel which begins to. Rotate giving rise to mixing of the contents , Heating can be incorporated where the instrument is called Hot plate magnetic stirrer .

Powder Flow and compaction :-

The main differences between Liquid and powder Rheology .

- The powder mass consist of a discrete particles with absenence of continuity .

- In powder certain factors influence their Rheological properties . e. g. particle shape , size and particle distribution .

FACTORS EFFECTING FLOW :-

- cohesion – By cohesion we mean the surface forces between particles .

- Rough and pitted surfaces – This may lead to inter – particulate friction .

- Irregular particles – which may lead to inter lock bridging or arching .

Cohesion :-

Definition – it is the stress necessary . to shear a bed of powder under condition of zero normal loads ,

cohesion may be measured by one of the following :-

- Shear cell :  it consist of two segments with atop Lid , the powder is packed carefully into the two segments and the lid is placed and loaded normally with varying Loads , the force required to shear the bed is measured and the shear stress can be found from the equation shear stress = force / cross sectioned area of the bed .

- Rote of sieving : cohesion of powder may affect the rate of sieving by forming aggregates or bridges .

- Mobility test : cohesion of fine powder (particles) is more than coarser ones , thus the addition of coarse fractions may reduce the cohesion of the powder , the minimum quantity of a coarse component to the material to make it free flowing is bases of the mobility test .

- The angle of repose : It can be measured by one of the following methods .

- By allowing the material to flow through a funnel or an orifice on a horizontal surface , this is only applicable to free flowing materials.

- An open ended cylinder resting on a horizontal surface is filled with powder , the powder will form a heap , the angle of which is the angle of repose .

EVAPORATION :-

Evaporation :- In pharmacy , evaporation signifies the process of applying heat to drive off a portion of the Liquid as a rapor . such a Liquid may consist of a solution of a nonvolatile substance in water , alcohol or other solvent . The purpose of evaporation is to consent vote the solution or to obtain the solute in dry from .

Evaporation may be hastened by :-

- using a shallow evaporating dish which exposes a Large a surface of the solution as possible .

- stirring the solution constantly which allows the free escape of vapor and ensure the even heating of the entire quantity of Liquid .

- using sufficient heat .

- Reducing atmospheric pressure .

Distillation :- Is the separation of the constituents of a Liquid mixture by vaporization and subsequent condensation of the vapors . such a separation is made possible by the different boiling points of the various volatile constituents .

(Evaporators should designed to give) :-

- Maximum heat transfer to the Liquid .

- Largest possible area .

- Reduce boundary Layers .

- Suitable temperature gradient .

- Turbulent flow conditions .

FACTORS AFFECTING EVAPORATION:-

- Temperature – Affects the rate of evaporation scientifically but , at the same time affects the drug constituents specially thermo labile materials Leading to its degradation e. g. alkaloids and Glycosides , hormones , antibiotics and enzymes . where freez drying method is carried It should be borne in mind that exposure to high temperature for short period is less destructives of active ingredients than expose to Lower temperature for a Long period , thus film evaporators are better than an evaporating pan .

- Tyne of product required . e. g. evaporating pan may produce both Liquid and dry products while film evaporators produces Liquid products only .

- Concentration – when a Liquid is concentrated , increases solid contents , which Lead to – solid may deposit decreases the heat transfer , so turbulent flow condition may reduce these problem .

Evaporators :-

These are classified into 3 main groups according to the form of movement .

- Natural circulation evaporators .

- Forced circulation evaporators .

- Film evaporators .

1- Natural circulation evaporation :- In this type the Liquid movement result from convection currents set up by the heating process . e. g.

a. Evaporating pan :- It is very simple , consist of a Jacketed hemispherical or shallow pan made from stain – less steel , steam circulate in the Jacket . The shape of the pan gives the best surface – volume ration for heating and Largest area for vapor removal .

The pan may be tilted to empty the product or supplied with an outlet at the bottom .

| Advantages | Disadvantages |
| --- | --- |
| 1- cheap and simple | 1- poor heat transfer . |
| 2-easy to construct ,to use , clean and to maintain . | 2- solids may deposite on the surface . |
|  | 3- Faming may occur during boiling . |
|  | 4- The bulk of the Liquid is heated all the time . |
|  | 5- Limited heating surface . |
|  | 6- The pan is open , saturation of the vapour may occur . |
|  | 7- it is only suitable for thermo – Labile solution and extracts . |

b- Evaporating still :- The evaporating pan may be supplied with a cover connecting it to a condenser so that the Liquid is distilled off , such evaporators are commonly known as 'stills' , The cover is designed to be easily removed for cleaning and removal of the product .

c- short tubes Evaporators : It consist of cylinder containing a No of tubes up to 1000 tubes each of 1-2 meter in Length and 40-80 mm. in diameter , heated by steam to boiling and this will increase the surface area of heating .

2- Forced circulation Evaporators:-  The natural circulation evaporators can be modified by adding a form of mechanical agitation to get forced circulation The agitators used are either as propeller or paddle agitator or a stirring rod .

  - Agitated evaporating pan :- The pan can be supplied with 1- stirring rod. 2- A mechanical operated agitator (propeller or puddle) .

  - Agitated short tube evaporators :- A mechanical agitator may introduced at the bottom of the short tube evaporator .

- Film evaporators :- In the previous evaporators Liquor is boiled in a vessel and the vapour escaps as bubbles from the bulk of the Liquid . In film E. the materials are spread as a thin film over the heated surface . leeching to an increase in heat transfer and recppid evaporation , the film may be formed naturally or mechanically

A – Long tube evaporators:- It consist of .

    - Heating unite composed of steam heated long tubes about 7 meter in length and 50 mm. in diameter .

    - Separator : To separat the concentrated Liquid to a reciver and vapour to a condenser .

There are two types of Long tube evaporators .

a- climbing .       b. Falling

B. wiped film evaporators :- (Rotary) – It consist of a cylindrical vessel supplied with a rotar having bladdes with a clearance of 1 mm. between the tips of the bladdes and the inner wall of the tube , when the Liquor is introduced at the top of the vessel m it will be spread as a film over the preheated walls of the vessels by the action of the rotar , the Liquid evaporates allows the vapour to pass to the condenser and the conc. Liquid or powder to be connected at the bottom of the vessel . The Loss of heat in evaporation procedure can be reduced by certain modification of the evaporators .

- Multiple effect evaporation .

- Vapour recompression .

- Reduced pressure .

Condenser :- A condenser is identical loan evaporator with the objectives reversed .

In evaporator – vapour (stem) is used to heat Liquid .

In condenser – Liquid (water) is used to cool vapour condensers can be divided into 2 main groups .

- Indirect (surface) condensers .

- Direct condensers .

Evaporation may be hastened by :-

- Using a shallow evaporating dish which exposes a Large a surface of the solution as possible .

- Stirring the solution constantly which allows the free escape of vapors and ensure the even heating of the entire quantity of Liquid.

- Using sufficient heat .

- Reducing atmospheric pressure .

Distillation :- Is the separation of the constituents of a Liquid mixture by vaporization and subsequent condensation of the vapors . such a separation is made possible by the different boiling points of the various volatile .

DRYING:-

Definition – Drying is the final removal of Liquid from solids by thermal vaporization Leaving a dry solid

the dry solid means that it is in equilibrium with the atmosphere and does not mean always a bone – dry solid .

Objectives :-

Drying is important for nearly each pharmaceutical process for the following reasones .

  - To increase the stability of moisture sensitive materials .

  - To increase the flow and mixing of solids .

  - To increase the rate of milling .

  - To decrease the cost of transport .

In certain cases the process may be the drying from dilute solution by sublimation drying known as freeze – drying [Factors affecting the rate of drying] .

- Temperature : The rate of drying is positively proportional with temperature .

- Relative humidity .it is the ratio between the humidity of air and the sati humidity HR=H/Hs /

Humidity of air – is the mass of water vapour present in a unite mass of dry air .

Saturated humidity – Is the mass of water vapour present in a unite mass of sat . air .

In both cases it should be in a specific pressure and temperature .

- surface area – Drying increases as the S.A.

- Air Flow .

- The thickness of the solid to be dry ed .

- Agitation .

- Tendency of the solid to shrink – which decreases the ability for drying .

Dryers

For dil sol and suspension  		               For solids

Thin Film                spray

Drum dryer           spray dryer

Convection                conduction                  Radiation

Tray (shelf)                             oven

Tunnel

Rotary

Fluidized bed

Dram Dryer :-

This consist of a drum heated internally by steam , the drum is rotated on its Longitudinal axis , a thin Film of the suspension is sprayed on it's surface by a special sprader , and the solid material is scraped by a knife the rate of drying can be controlled by speed of rotation and temperature . The dryer may be enclosed in a vaccum Jaket to facilitate the drying rate . such dryers are used for thermo labile products .

Spray dryer :-

In these dryers the suspension is sprayed by an atomizer to small droplets into vessel heated with steam of hot air allowing the droplets to dry to small solid products Like milk , coffe , a well as for chemicals e. g. extracts . citric acid , hexamine , starch and other products like soap detergents Advantages :-

- Rapid evaporation .

- Short heating period .

- Homogenous particles , uniform and spherical .

- Low Labour cost .

Disadvantages :-

- Expensive and bulky equipment .

- Great Loss of heat .

- Used only for Large scale .

Dryers for solids : can be classified according to the principle method of heat transfer .

1- Convection dryers :

a- Tray or Shaft dryer :- This is consist of .

- A cabinet supplied with heater at the bottom .

- Fan to improve heat and humidity transfer (forced circulation) .

- In advanced forms – hated air is circulated across the material .

- The shelves are arranged alternatively helping air flow .

- Continuous removing the trays nearest to the source of heat replacing it with fresh trays to overcome , over heating . This type of dryers . is used for chemicals , powders and granules ..

b. Tunnel dryers :- This resemble the tray dryer with the following differences .

- The dryer consist of a Long tunnel with heated air entering at one end and : Leaving at the opposite end .

- The material to be dried is moving and not stationary by .

- Either the mat is placed on trays on rails . Fresh tray is pushed into the inlet at intervals and a dry tray is removed from the other end .

- Or the mat is placed on a conveyer which moves mechanically at a controlled speed .

- The temperature and humidity is controlled , this dryer operates continuously . It is used for large scale production .